IMU-Guided Cross-Attention

for Robust Visual-Inertial Drone Localisation

in Low-Texture Environments

Minor Project Research Plan  ·  SRM Institute of Science and Technology  ·  2026

Sai Ganeshan M  ·  Nikhil Bhosale  ·  Adit Lakshman  ·  Krishiv  ·  Abhineet

| Version | Status | Date |
| --- | --- | --- |
| 1.0 | Minor Project Plan | March 2026 |

# 1. Abstract

Drones flying without GPS must figure out where they are using only a camera and an IMU sensor. Existing methods track corners and edges in camera images to estimate motion — but they break completely in environments with no texture, such as white corridors, tunnels, or dusty rooms. This project proposes a simple neural network that uses IMU sensor readings to guide where the camera should look in the image. When the IMU feels a force, the model focuses its visual attention on the image region that caused it. This is done using a Cross-Attention mechanism — a standard deep learning component available in PyTorch. The system is trained on real drone flight data from the EuRoC MAV dataset and compared against a simpler baseline that just stacks image and IMU features together. The key question: does guiding visual attention with physical motion improve accuracy?

# 2. What Problem Are We Solving?

## 2.1 The Short Answer

One Sentence Summary A drone needs to know where it is — without GPS — in an environment with no visual landmarks.

## 2.2 The Longer Explanation

Outdoors, drones use GPS for positioning. GPS works by receiving signals from satellites. Inside a building, tunnel, or underground space — there are no satellite signals. The drone is now completely on its own.

Without GPS, drones use two sensors together:

- A camera — to see the environment

- An IMU — to feel motion (acceleration and rotation)

The standard method is called Visual Inertial Odometry or VIO. It works like this: find distinctive corners and edges in the camera image, track them as the drone moves, and calculate how far the drone has moved based on how those points shifted.

Where VIO Breaks VIO requires distinctive visual features to track. In these environments it fails completely: • White corridors with uniform walls • Concrete tunnels with no patterns • Dusty or smoky rooms • Dark environments • Industrial warehouses with smooth surfaces These are exactly the environments where drones are needed most — search and rescue, building inspection, disaster zones.

## 2.3 The Second Problem — IMU Drift

Even without a camera, the IMU can estimate motion by measuring acceleration and integrating it over time. The problem is that every cheap IMU chip has a tiny built-in error. When you integrate that error over 60 seconds, it grows quadratically. The drone thinks it is in one place but is actually several metres away.

Concrete Example A 0.001 m/s² IMU bias, integrated twice over 60 seconds, produces roughly 1.8 metres of position error. Without GPS, there is nothing to correct it.

## 2.4 The Physical Insight — Why This Project Makes Sense

Key Insight What the IMU feels and what the camera sees are connected. If the drone suddenly lurches to the left, something on the left side of the camera image caused it. A wall, a gust from an opening, or a narrow passage — they all create both a physical force and a visible signal at the same time. Current systems treat camera and IMU as completely separate inputs and just stack their features together. This ignores the physical relationship between them. This project asks: what if the IMU told the camera where to look?

# 3. How Is This Different From What Exists?

## 3.1 Classical VIO — The Current Standard

Systems like ORB-SLAM3 and OpenVINS are the current best methods for GPS-denied drone navigation. Here is exactly what they do:

Step 1: Camera captures image Step 2: Find corners and edges in the image (feature detection) Step 3: Find the same corners in the next image (feature tracking) Step 4: Calculate how the drone moved from the shift in those corners Step 5: Use IMU to fill in gaps between camera frames

The Problem With This Approach If there are no corners or edges to find — the entire system collapses. No features = no tracking = no localisation. The drone is lost.

## 3.2 Standard Deep Learning Fusion — The Current Learned Baseline

Several papers have replaced hand-crafted feature tracking with neural networks. Most of them do this:

# What most learning-based systems do imu_features = LSTM(imu_window) # encode IMU image_features = CNN(camera_frame) # encode image combined = concat(imu_features, # just stack them image_features) displacement = LinearHead(combined) # predict motion

This concatenation approach works better than classical VIO in some cases. But it still treats IMU and camera as two separate, unrelated streams of data. It does not use the physical relationship between them.

## 3.3 This Project — Cross-Attention Fusion

This project replaces the concatenation step with a Cross-Attention mechanism:

# What this project does imu_embedding = LSTM(imu_window) # encode IMU image_patches = CNN(camera_frame) # encode image as 49 patches # Cross-Attention: IMU asks 'which image patch caused this force?' fused = CrossAttention( query = imu_embedding, # IMU is the question keys = image_patches, # image regions are the answers values = image_patches ) displacement = LinearHead(fused) # predict motion

What This Means In Simple Words Instead of stacking IMU and image features together, the IMU embedding acts as a question: 'I just felt a force — which part of the image caused it?' The attention mechanism searches all 49 image patches and assigns a weight to each one. High weight means that patch is relevant to the current motion. The final prediction is made from the attended visual features — the parts of the image most related to what the IMU just felt.

## 3.4 Side-by-Side Comparison

| Property | Classical VIO | Concat Fusion | Ours (Cross-Attention) |
| --- | --- | --- | --- |
| Works without texture | No — fails | Partially | Better — uses semantics |
| Models IMU-camera relationship | No | No | Yes — via attention |
| Needs visual feature points | Yes — required | Partially | No |
| Complexity | Low | Low | Low — standard PyTorch |
| Publishable novelty | No | No | Yes — if ablation wins |

# 4. System Architecture — How It Is Built

The entire model has four components. Each one is simple, standard, and available in PyTorch. There is nothing custom or complex here.

Architecture Overview Component 1 — IMU Encoder: A 2-layer LSTM reads 100 IMU samples and produces a 256-number summary of how the drone is moving. Component 2 — Image Encoder: A frozen pretrained MobileNetV3 reads the camera frame and produces 49 image patches, each described by 256 numbers. Component 3 — Cross-Attention: The IMU summary (the query) attends to all 49 image patches. Output is a 256-number fused feature that captures both physical motion and visual context. Component 4 — Prediction Head: A small 3-layer network takes the fused feature and outputs Δx, Δy, Δz — how far the drone moved.

## 4.1 Component 1 — IMU Encoder

The IMU measures acceleration (x, y, z) and rotation rate (x, y, z) at 200 times per second. This gives 6 numbers per timestep. We take the last 100 readings as a window (0.5 seconds of motion history) and feed them into an LSTM.

Input shape: [batch, 100 timesteps, 6 features] (acceleration xyz + gyroscope xyz) LSTM: 2 layers, hidden size 256 Output: [batch, 256] — a single vector summarising drone motion — this becomes the Cross-Attention QUERY

## 4.2 Component 2 — Image Encoder

Each camera frame is processed by MobileNetV3-Small, a lightweight CNN pretrained on ImageNet. The backbone weights are frozen — meaning we do not train them. This saves time, prevents overfitting, and means we only need to train the fusion and prediction parts.

Input: camera frame [batch, 3, 224, 224] MobileNetV3: frozen pretrained backbone outputs feature map [batch, 576, 7, 7] Projection: 1x1 Conv → [batch, 256, 7, 7] Reshape: [batch, 49, 256] 49 spatial patches, each 256-dimensional these become the Cross-Attention KEYS and VALUES

## 4.3 Component 3 — Cross-Attention Fusion

This is the central idea. PyTorch has a built-in MultiheadAttention module that does exactly what is needed. The IMU vector is the query. The 49 image patches are the keys and values.

query = imu_embedding [batch, 1, 256] keys = image_patches [batch, 49, 256] values = image_patches [batch, 49, 256] attention_weights = softmax( query @ keys.T / sqrt(256) ) → shape [batch, 1, 49] → 49 weights, one per image patch → high weight = that patch is relevant to current IMU state fused = attention_weights @ values → shape [batch, 1, 256] → visual features weighted by physical relevance

What The Attention Weights Mean Each of the 49 weights corresponds to one spatial region of the camera image (arranged in a 7×7 grid). When the attention weight for a patch is high, the model is saying: that part of the image is most related to the physical force the IMU just detected. These weights can be visualised as a heatmap overlaid on the camera image — giving a visual and interpretable explanation of what the model learned.

## 4.4 Component 4 — Displacement Prediction Head

A simple 3-layer fully connected network takes the fused 256-dimensional feature and predicts the drone's displacement since the last camera frame. This is the ground truth the network is trained against.

Input: fused feature [batch, 256] Linear(256 → 64) → GELU Linear(64 → 64) → GELU Linear(64 → 3) Output: [Δx, Δy, Δz] in metres displacement since previous camera frame

# 5. Dataset

## 5.1 EuRoC MAV Dataset

The EuRoC MAV dataset is the standard benchmark for visual-inertial drone odometry. It is free, publicly available, and used in virtually every VIO paper published since 2016. It contains 11 recorded drone flights inside a building with a motion capture system that provides millimetre-accurate ground truth position.

| Property | Details |
| --- | --- |
| Camera | Stereo camera at 20 Hz — we use left camera only |
| IMU | 200 Hz — acceleration + gyroscope, 6 values per timestep |
| Ground truth | 200 Hz from motion capture — millimetre accuracy |
| Number of sequences | 11 indoor flight sequences |
| Licence | CC BY — free to use for research |
| Download | https://rpg.ifi.uzh.ch/mav_datasets.html |

## 5.2 Data Splits

- Train: V1_01, V1_02, V1_03, V2_01, MH_01, MH_03 — used to train the model

- Validation: V2_02, MH_02 — used to tune hyperparameters

- Test: V2_03, MH_04 — used only at the end for final results

Why Keep Test Data Separate? The test sequences are never seen during training or tuning. This ensures the ATE numbers in your paper reflect how the model performs on completely new data — not data it already saw. A paper that reports results on training data is not trustworthy.

## 5.3 Label Generation

Training labels are computed directly from ground truth positions. For each pair of consecutive camera frames, the displacement is simply the difference in position:

# For each consecutive camera frame pair: delta_p = position_at_time_t1 - position_at_time_t0 # This gives: [Δx, Δy, Δz] in metres # This is what the network is trained to predict # No wind labels needed # No Butterworth filter needed # No physics residual decomposition needed # Just position difference — simple and clean

# 6. Full Implementation

## 6.1 Setup

# Create environment conda create -n imu_attn python=3.10 conda activate imu_attn # Install dependencies pip install torch torchvision pip install numpy pandas scipy matplotlib tqdm pip install rosbags # for reading EuRoC bag files pip install pillow # for image loading

## 6.2 Data Loader

import numpy as np import pandas as pd from pathlib import Path from PIL import Image import torch from torch.utils.data import Dataset import torchvision.transforms as T class EuRoCDataset(Dataset): def __init__(self, seq_paths, window=100): self.samples = [] self.window = window self.transform = T.Compose([ T.Resize((224, 224)), T.ToTensor(), T.Normalize([0.485,0.456,0.406], [0.229,0.224,0.225]) ]) for path in seq_paths: self._load_sequence(Path(path)) def _load_sequence(self, path): imu = pd.read_csv(path/'imu0/data.csv', names=['t','ax','ay','az','gx','gy','gz'], skiprows=1) gt = pd.read_csv(path/'state_groundtruth_estimate0/data.csv', names=['t','px','py','pz','qw','qx','qy','qz', 'vx','vy','vz','bwx','bwy','bwz','bax','bay','baz'], skiprows=1) cam = pd.read_csv(path/'cam0/data.csv', names=['t','filename'], skiprows=1) cam_times = cam['t'].values for i in range(1, len(cam_times)): t0, t1 = cam_times[i-1], cam_times[i] # IMU window — last 100 readings before t1 w = imu[imu['t'] <= t1].tail(self.window) if len(w) < self.window: continue imu_data = w[['ax','ay','az','gx','gy','gz']].values # Ground truth displacement g1 = gt.iloc[(gt['t']-t1).abs().argsort()[:1]] g0 = gt.iloc[(gt['t']-t0).abs().argsort()[:1]] dp = (g1[['px','py','pz']].values - g0[['px','py','pz']].values).squeeze() img_path = path/'cam0/data'/cam.iloc[i]['filename'] self.samples.append({ 'imu': imu_data.astype(np.float32), 'dp': dp.astype(np.float32), 'img': str(img_path) }) def __len__(self): return len(self.samples) def __getitem__(self, idx): s = self.samples[idx] img = Image.open(s['img']).convert('RGB') return (torch.tensor(s['imu']), self.transform(img), torch.tensor(s['dp']))

## 6.3 The Full Model — Both Versions

import torch.nn as nn import torchvision.models as models # ───────────────────────────────────────────────────────────── # YOUR MODEL: Cross-Attention Fusion # ───────────────────────────────────────────────────────────── class CrossAttentionNet(nn.Module): def __init__(self): super().__init__() # Component 1 — IMU Encoder self.lstm = nn.LSTM(6, 256, 2, batch_first=True) self.imu_norm = nn.LayerNorm(256) # Component 2 — Image Encoder (frozen) mob = models.mobilenet_v3_small(pretrained=True) self.img_net = mob.features self.img_proj = nn.Conv2d(576, 256, 1) for p in self.img_net.parameters(): p.requires_grad = False # frozen — do not train # Component 3 — Cross-Attention self.attn = nn.MultiheadAttention( embed_dim=256, num_heads=4, batch_first=True) self.norm = nn.LayerNorm(256) # Component 4 — Prediction Head self.head = nn.Sequential( nn.Linear(256, 64), nn.GELU(), nn.Linear(64, 64), nn.GELU(), nn.Linear(64, 3) ) def forward(self, imu, img): # IMU → Query _, (h, _) = self.lstm(imu) q = self.imu_norm(h[-1]).unsqueeze(1) # [B,1,256] # Image → Keys and Values f = self.img_proj(self.img_net(img)) # [B,256,7,7] kv = f.flatten(2).transpose(1,2) # [B,49,256] # Cross-Attention fused, weights = self.attn(q, kv, kv) # [B,1,256] fused = self.norm(fused.squeeze(1)) # [B,256] return self.head(fused), weights # [B,3], [B,1,49] # ───────────────────────────────────────────────────────────── # BASELINE: Simple Concatenation # ───────────────────────────────────────────────────────────── class ConcatNet(nn.Module): def __init__(self): super().__init__() self.lstm = nn.LSTM(6, 256, 2, batch_first=True) mob = models.mobilenet_v3_small(pretrained=True) self.img_net = mob.features self.img_pool = nn.AdaptiveAvgPool2d(1) self.img_proj = nn.Linear(576, 256) for p in self.img_net.parameters(): p.requires_grad = False self.head = nn.Sequential( nn.Linear(512, 64), nn.GELU(), nn.Linear(64, 64), nn.GELU(), nn.Linear(64, 3) ) def forward(self, imu, img): _, (h, _) = self.lstm(imu) imu_f = h[-1] # [B,256] img_f = self.img_pool(self.img_net(img)).flatten(1) img_f = self.img_proj(img_f) # [B,256] combined = torch.cat([imu_f, img_f], 1) # [B,512] return self.head(combined) # [B,3]

## 6.4 Training Loop

from torch.utils.data import DataLoader import torch def train_model(model, train_set, val_set, epochs=50): loader = DataLoader(train_set, batch_size=64, shuffle=True, num_workers=2) val_loader = DataLoader(val_set, batch_size=64) # Only train parameters that are not frozen opt = torch.optim.Adam( filter(lambda p: p.requires_grad, model.parameters()), lr=1e-4, weight_decay=1e-4 ) scheduler = torch.optim.lr_scheduler.StepLR( opt, step_size=20, gamma=0.5) loss_fn = torch.nn.MSELoss() device = 'cuda' if torch.cuda.is_available() else 'cpu' model = model.to(device) for epoch in range(epochs): model.train() train_loss = 0 for imu, img, gt in loader: imu, img, gt = imu.to(device), img.to(device), gt.to(device) opt.zero_grad() if isinstance(model, CrossAttentionNet): pred, _ = model(imu, img) else: pred = model(imu, img) loss = loss_fn(pred, gt) loss.backward() torch.nn.utils.clip_grad_norm_(model.parameters(), 1.0) opt.step() train_loss += loss.item() scheduler.step() print(f'Epoch {epoch+1:3d} ¦ Train: {train_loss/len(loader):.4f}')

## 6.5 ATE Evaluation + Low-Texture Test

import numpy as np import cv2 def compute_ate(model, dataset, blur=False, device='cuda'): """ Accumulate predicted displacements into a trajectory. Compare against ground truth using ATE. If blur=True, apply Gaussian blur to simulate low-texture. """ loader = DataLoader(dataset, batch_size=1, shuffle=False) model.eval() pred_pos = np.zeros(3) gt_pos = np.zeros(3) pred_traj, gt_traj = [pred_pos.copy()], [gt_pos.copy()] with torch.no_grad(): for imu, img, dp_gt in loader: if blur: # Simulate featureless environment img_np = img.squeeze().permute(1,2,0).numpy() img_np = cv2.GaussianBlur(img_np, (25,25), 10) img_np = cv2.convertScaleAbs(img_np, alpha=0.3, beta=128) / 255.0 img = torch.tensor(img_np).permute(2,0,1) img = img.unsqueeze(0) imu = imu.to(device); img = img.to(device) if isinstance(model, CrossAttentionNet): dp_pred, _ = model(imu, img) else: dp_pred = model(imu, img) pred_pos = pred_pos + dp_pred.cpu().numpy().squeeze() gt_pos = gt_pos + dp_gt.numpy().squeeze() pred_traj.append(pred_pos.copy()) gt_traj.append(gt_pos.copy()) pred = np.array(pred_traj) gt = np.array(gt_traj) pred = pred + (gt[0] - pred[0]) # align start ate = np.sqrt(np.mean(np.sum((pred-gt)**2, axis=1))) return float(ate) # Run both experiments ate_normal = compute_ate(model, test_set, blur=False) ate_blurred = compute_ate(model, test_set, blur=True) print(f'Normal ATE: {ate_normal:.3f} m') print(f'Low-Texture ATE:{ate_blurred:.3f} m')

# 7. Experiments and Expected Results

## 7.1 The Critical Ablation Table

This is the most important table in your paper. It directly answers the question: does cross-attention improve over concatenation? If it does — you have a publishable result.

| Model | Normal ATE (m) | Low-Texture ATE (m) | Params |
| --- | --- | --- | --- |
| Raw IMU only (double integration) | ~2.10 | ~2.10 | — |
| LSTM IMU only (no camera) | ~1.45 | ~1.45 | 2M |
| Concatenation — baseline | ~0.98 | ~1.80 | 6M |
| Cross-Attention — ours | ~0.71 | ~1.05 | 6.2M |

How To Read This Table The numbers shown above are expected estimates based on similar published work. Your actual numbers will differ. What matters is the PATTERN: • Concatenation should beat LSTM-only (camera helps) • Cross-Attention should beat Concatenation (attention helps) • The gap should be larger in Low-Texture than Normal conditions If cross-attention beats concatenation in both columns — submit to IEEE Sensors Journal.

## 7.2 Attention Visualisation

Save the attention weights from the cross-attention layer and overlay them on the original camera frame as a heatmap. This is one of the most valuable figures in your paper because it makes the model's behaviour interpretable and human-readable.

import matplotlib.pyplot as plt import numpy as np def visualise_attention(model, imu, img, original_img): model.eval() with torch.no_grad(): _, weights = model(imu.unsqueeze(0), img.unsqueeze(0)) # weights: [1, 1, 49] → reshape to 7×7 grid attn_map = weights.squeeze().reshape(7, 7).numpy() # Resize to match original image (224×224) import cv2 attn_resized = cv2.resize(attn_map, (224, 224)) # Overlay on original image fig, axes = plt.subplots(1, 2, figsize=(10, 4)) axes[0].imshow(original_img) axes[0].set_title('Camera Frame') axes[1].imshow(original_img) axes[1].imshow(attn_resized, alpha=0.6, cmap='jet') axes[1].set_title('IMU-Guided Attention') plt.savefig('attention_heatmap.png', dpi=150) plt.show()

What Good Attention Looks Like When the drone moves forward: attention should be high near the centre of the image (approaching objects). When the drone turns left: attention should be high on the left side (objects coming into view). When the drone is hovering still: attention should be spread evenly (no dominant direction). If the attention maps follow this pattern — you have evidence the model is physically reasoning, not just memorising patterns. This is your key paper figure.

# 8. Four-Week Execution Plan

| Week | Days | Task | Deliverable |
| --- | --- | --- | --- |
| Week 1 | 1–2 | Download EuRoC. Implement data parser. Verify: print one sample shape (imu, img, delta_p). Ensure synchronisation works. | Working DataLoader. One batch loads correctly. |
| Week 1 | 3–5 | Implement both models: ConcatNet and CrossAttentionNet. Verify forward pass runs without error. Count parameters. | Both models forward pass. Parameter counts confirmed. |
| Week 2 | 1–3 | Train ConcatNet on train sequences. Monitor loss curve. Tune learning rate if loss does not decrease. | Trained ConcatNet checkpoint. Loss curve plot. |
| Week 2 | 4–5 | Train CrossAttentionNet. Use same hyperparameters as ConcatNet for fair comparison. | Trained CrossAttentionNet checkpoint. |
| Week 3 | 1–2 | Evaluate both models on test sequences. Compute ATE. Build result table. | ATE table: normal conditions. |
| Week 3 | 3–4 | Run low-texture experiment with Gaussian blur on test images. Recompute ATE for both models. | ATE table: low-texture conditions. |
| Week 3 | 5 | Generate attention heatmap visualisations. Save 4–6 good examples. Run 3 random seeds. | Attention figures. Mean ± std results. |
| Week 4 | 1–3 | Write paper in IEEE 4-page two-column format. Write minor project report. | Paper draft. Report draft. |
| Week 4 | 4–5 | Finalise figures. Proofread. Submit minor project report. | Final submission. |

# 9. Publication Path

The table below shows where to submit based on your results. Be honest about what your numbers show.

| Venue | Tier | Condition For Submission | Deadline |
| --- | --- | --- | --- |
| IEEE Sensors Journal | Good mid-tier | Cross-attention beats concatenation in both normal and low-texture ATE | Rolling — submit anytime |
| IEEE Access | Mid-tier open access | Same — any positive result | Rolling |
| IROS 2026 Workshop | Good visibility | Any interesting result — workshops are more accepting | Check IROS 2026 website |
| Minor Project Report | Required | Always — this is your deliverable regardless of publication | Your institute deadline |

# 10. Literature Survey — Papers You Must Read

You must understand and cite these papers. Each one is explained in simple words and linked directly.

## 10.1 The Two Baselines You Are Beating

TLIO: Tight Learned Inertial Odometry (2020 · IEEE RA-L) The foundational learned odometry paper. Uses an LSTM on IMU windows to predict displacement, then feeds it into a Kalman filter. This is your IMU-only baseline. Read it to understand how displacement-based odometry works. Your LSTM encoder is directly inspired by this. Paper: https://ieeexplore.ieee.org/document/9134860 GitHub: https://github.com/CathIAS/TLIO

OpenVINS: A Research Platform for Visual-Inertial Estimation (2020 · ICRA) State-of-the-art classical VIO. Tracks feature points across frames and fuses with IMU. This is the system you are trying to improve upon in low-texture environments. Run it on your blurred test images and show it fails. That failure motivates your paper. Paper: https://docs.openvins.com GitHub: https://github.com/rpng/open_vins

## 10.2 Your Architectural Blueprint

AirIO: Learning Inertial Odometry with Enhanced IMU Feature Observability (2025 · IEEE RA-L) Proves that body-frame IMU encoding (keeping measurements in the drone's own coordinate frame rather than converting to world frame) gives 66.7% accuracy improvement. Your LSTM encoder should follow this approach — do not convert IMU to global frame before feeding it to the LSTM. Paper: https://arxiv.org/abs/2501.15659 GitHub: https://github.com/Air-IO/Air-IO

## 10.3 The Paper That Motivates Cross-Attention For Sensor Fusion

TransFuser: Imitation with Transformer-Based Sensor Fusion for Autonomous Driving (2021 · PAMI) Uses cross-attention to fuse LiDAR and camera features for autonomous cars. This is the closest architectural precedent to your method. A reviewer will mention this paper. Your response: TransFuser uses LiDAR-as-query for cars. You use IMU-physical-state-as-query for drones. Different modality, different domain, different physical meaning. Paper: https://arxiv.org/abs/2205.15997 GitHub: https://github.com/autonomousvision/transfuser

## 10.4 The Strongest Competitor You Must Compare Against

HDVIO2.0: Wind and Disturbance Estimation with Hybrid Dynamics VIO (2025 · IEEE T-RO) The most advanced visual-inertial system with wind estimation. Uses TCNs inside a full VIO pipeline. This is the state of the art you are working near. You are not trying to beat it overall — you are showing your simpler approach works better specifically in low-texture conditions where their feature-based system fails. Paper: https://arxiv.org/abs/2504.00969 GitHub: https://github.com/uzh-rpg/hdvio2.0

# 11. All Links — Quick Reference

DATASETS

EuRoC MAV Dataset (ETH Zurich) — primary dataset:  https://rpg.ifi.uzh.ch/mav_datasets.html

TUM-VI Dataset (TU Munich) — optional for extra evaluation:  https://cvg.cit.tum.de/data/datasets/visual-inertial-dataset

PAPERS

TLIO (IEEE RA-L 2020):  https://ieeexplore.ieee.org/document/9134860

AirIO (IEEE RA-L 2025):  https://arxiv.org/abs/2501.15659

TransFuser (CVPR/PAMI 2021):  https://arxiv.org/abs/2205.15997

HDVIO2.0 (IEEE T-RO 2025):  https://arxiv.org/abs/2504.00969

AirIMU (arXiv 2023):  https://arxiv.org/abs/2310.04874

GITHUB / CODE

TLIO implementation:  https://github.com/CathIAS/TLIO

OpenVINS — classical VIO baseline:  https://github.com/rpng/open_vins

AirIO implementation:  https://github.com/Air-IO/Air-IO

HDVIO2.0 implementation:  https://github.com/uzh-rpg/hdvio2.0

TransFuser implementation:  https://github.com/autonomousvision/transfuser

TOOLS

PyTorch — model training:  https://pytorch.org

rosbags — read EuRoC bag files:  https://github.com/rpng/rosbags

EuRoC dataset paper (IJRR):  https://rpg.ifi.uzh.ch/docs/IJRR17_Burri.pdf

IMU-Guided Cross-Attention  ·  Minor Project  ·  SRM Institute of Science and Technology  ·  2026

EuRoC MAV dataset is CC BY licensed  ·  All links verified March 2026